UNIT I

Semiconductor Diodes and Their Applications

Introduction to Electronics Engineering (IEE)

# Topic 1: Introduction to Electronics

## 1.1 What is Electronics?

Electronics is the branch of science and engineering that deals with the study of the flow of electrons through vacuum, gas, or semiconductors. It involves the design and application of circuits using these electron flows to perform specific functions.

An electronic circuit is a closed path formed by the interconnection of electronic components through which electric current flows to perform a specific function.

### Types of Electronics

- Analog Electronics

- Digital Electronics

- Power Electronics

- Microelectronics

- Optoelectronics

- Communication Electronics

# Topic 2: Atomic Structure and Semiconductor Fundamentals

## 2.1 Atomic Structure

An atom is the smallest particle of an element that retains the chemical properties of that element.

- Atomic structure refers to the structure of an atom comprising of a nucleus (centre) in which protons (positively charged) and neutrons (neutral) are present.

- The negatively charged particles called electrons revolve around the centre of the nucleus.

- Each permitted orbit has a fixed amount of energy. The larger the orbit (radius), the greater is the energy of electrons.

- If an electron gains additional energy (heat, light, etc.), it moves to a higher orbit. The atom is then in a state of excitation.

Key Formulae:

Atomic Weight: Number of Protons + Number of Neutrons

Atomic Number: Number of Protons = Number of Electrons in the atom

## 2.2 Semiconductor

In terms of their electrical properties, materials can be classified into three groups: conductors, semiconductors, and insulators.

- Semiconductor: A material whose electrical conductivity lies between that of a conductor and an insulator, and whose conductivity can be controlled by adding impurities (doping) or applying external fields.

- The behaviour of electrons in solids is governed by their arrangement in energy bands.

### Energy Bands

Conduction Band: The energy band where electrons are free to move and contribute to electrical conductivity.

Valence Band: The energy band where electrons are bound to atoms and do not contribute to conductivity.

Fermi Level: The highest energy level that an electron can occupy at absolute zero temperature. It lies between the valence band and conduction band.

### Most Common Semiconductors

- Semiconductor materials fall into two classes: single-crystal and compound.

- The three semiconductors most frequently used in electronic devices are: Germanium (Ge), Silicon (Si), and Gallium Arsenide (GaAs).

- In a pure silicon or germanium crystal, the four valence electrons of one atom form a bonding arrangement with four adjoining atoms — this is called covalent bonding.

## 2.3 Comparison of Materials: Conductors, Semiconductors, and Insulators

| Parameter | Conductor | Semiconductor | Insulator |
| --- | --- | --- | --- |
| Forbidden Energy Gap | Not Exist | Small (1 eV) | Large (>5 eV) |
| Resistivity | Low | Moderate | High |
| Flow of Current | Due to movement of free electrons | Due to movement of electrons and holes | Almost negligible but only due to free electrons |
| Charge Carriers in Conduction Band | Completely filled | Partially filled | Completely vacant |
| Charge Carriers in Valence Band | Almost vacant | Partially filled | Completely filled |
| Example | Copper, Aluminium, Graphite | Silicon, Germanium, Arsenic | Paper, Rubber, Glass, Plastic |
| Applications | Conducting wires, Transformers, Electrical cords | Diodes, Transistors, Optocouplers | Sports equipment, Home appliances |

## 2.4 Types of Semiconductors

### A. Intrinsic Semiconductors

- Pure-form semiconductor materials without any significant impurities. Examples: Silicon (Si) and Germanium (Ge).

- In an intrinsic semiconductor, the number of electrons in the conduction band equals the number of holes (empty states in the valence band).

- Conductivity is low at room temperature and increases with temperature.

### B. Extrinsic Semiconductors (Doped)

Extrinsic semiconductors are created by adding specific impurities (called dopants) to an intrinsic semiconductor to enhance its electrical conductivity.

### N-Type Semiconductor

- Created by adding pentavalent impurity elements (5 valence electrons) such as Antimony (Sb), Arsenic (As), and Phosphorus (P).

- The impurity atoms donate a free electron, so they are called donor atoms.

- Majority carriers: Electrons | Minority carriers: Holes

### P-Type Semiconductor

- Created by adding trivalent impurity elements (3 valence electrons) such as Boron (B), Gallium (Ga), and Indium (In).

- The impurity atoms accept an electron, creating a hole; they are called acceptor atoms.

- Majority carriers: Holes | Minority carriers: Electrons

The controlled addition of dopants allows precise control over the electrical properties of semiconductors, making them essential in devices like diodes, transistors, and integrated circuits.

## 2.5 Intrinsic vs. Extrinsic Semiconductors — Comparison Table

| Property | Intrinsic Semiconductor | Extrinsic Semiconductor (n-type & p-type) |
| --- | --- | --- |
| Purity | Pure (no impurities) | Doped with impurities |
| Conductivity | Low at room temperature | Higher due to the presence of free charge carriers |
| Charge Carriers | Equal number of electrons and holes | Majority and minority carriers (electrons or holes) |
| Effect of Temperature | Conductivity increases with temperature | Conductivity depends on doping and temperature |
| Types | Not applicable | n-type (electron majority) and p-type (hole majority) |
| Energy Band Structure | No additional energy levels | Donor or acceptor energy levels present |

## 2.6 Majority and Minority Carriers

When a semiconductor is doped, one type of charge carrier becomes dominant:

| Material Type | Carrier Information |
| --- | --- |
| N-Type | Majority Carrier: Electrons ¦ Minority Carrier: Holes |
| P-Type | Majority Carrier: Holes ¦ Minority Carrier: Electrons |

# Topic 3: PN Junction Diode

## 3.1 Introduction

- The PN junction is the fundamental building block of semiconductor devices such as diodes, transistors, and integrated circuits.

- The PN junction diode is created by simply joining an n-type and a p-type semiconductor material together.

### Operating Conditions of a Diode

- No Bias — No external voltage applied (VD = 0)

- Forward Bias — Positive terminal to p-region, negative to n-region

- Reverse Bias — Positive terminal to n-region, negative to p-region

Bias: The application of an external voltage across the two terminals of the device to extract a response.

## 3.2 PN Junction: No Bias Condition

- Electrons from the n-region diffuse into the p-region; holes from the p-region diffuse into the n-region. Movement continues until equilibrium is established.

- Electrons and holes recombine near the junction, creating a depletion region — a region with no free charge carriers. Only immobile ions remain (negative ions on p-side, positive ions on n-side).

- An electric field develops across the depletion region, opposing further diffusion.

- In the absence of applied bias, the net flow of charge in any direction is zero.

### Characteristics of the Depletion Region

Width: Depends on doping concentration. Heavily doped regions have narrower depletion regions.

Charge Distribution: p-region contains negatively charged acceptor ions; n-region contains positively charged donor ions.

Electric Field: Points from positively charged n-region to negatively charged p-region.

### Barrier Potential (Built-in Potential)

The electric field creates a potential barrier (Vb) that prevents further diffusion of charge carriers:

- Silicon: Vb = 0.7 V

- Germanium: Vb = 0.3 V

## 3.3 Reverse Bias

When the positive terminal is connected to the n-region and the negative terminal to the p-region:

- Potential barrier is increased

- Depletion region widens

- Charge carriers cannot cross the junction

- Very little current flows (due to minority charge carriers only)

- If the reverse voltage exceeds a critical value, breakdown can occur through Zener or Avalanche breakdown

### Reverse Saturation Current (Is)

Also called leakage current. It is caused by thermally generated minority carriers swept across the junction by the built-in electric field.

- Very small in magnitude

- Almost constant with reverse voltage

- Strongly increases with temperature

## 3.4 Forward Bias

When the positive terminal is connected to the p-region and the negative terminal to the n-region:

- Potential barrier is reduced

- Charge carriers can cross the junction

- Electrons move toward the p-region; holes move toward the n-region

- Results in significant forward current flow

- In silicon diodes, forward voltage must be approximately 0.7 V for significant conduction

## 3.5 Shockley's Diode Equation

The general current-voltage relationship for a semiconductor diode is given by Shockley's equation:

ID = Is (e(VD / n*VT) - 1)

| Symbol | Definition |
| --- | --- |
| I_D | Diode current |
| I_s | Reverse saturation current |
| V_D | Applied forward-bias voltage across the diode |
| n | Idealist factor: n = 1 for Germanium; n = 2 for Silicon |
| V_T | Thermal Voltage = k * T_K / q |
| k | Boltzmann's constant = 1.38 × 10⁻²³ J/K |
| T_K | Absolute temperature in kelvins = 273 + Temperature (°C) |
| q | Magnitude of electronic charge = 1.6 × 10⁻¹⁹ C |

## 3.6 Current-Voltage (I-V) Characteristics

### Forward Bias Region

- When voltage exceeds the threshold (0.7 V for silicon, 0.3 V for germanium), current increases exponentially.

- Diode conducts current easily once threshold is crossed.

### Reverse Bias Region

- Only a very small reverse saturation current (Is) flows.

- If the reverse voltage exceeds the breakdown voltage, the diode conducts large currents — this may permanently damage regular diodes.

Knee Voltage: The minimum forward voltage after which the forward current starts increasing exponentially. For Si: ~0.7 V; for Ge: ~0.3 V.

## 3.7 Diode Resistance

### Static Resistance (DC Resistance) — RD

Defined at a specific operating point on the I-V characteristics:

RD = VD / ID

### Dynamic Resistance (AC Resistance) — rd

Calculated using the slope of the I-V curve at the operating point:

rd = ΔVd / ΔId = n*VT / ID

At room temperature (27°C), V_T ≈ 26 mV, and for n = 1:

rd = 26 mV / ID

## 3.8 Temperature Effects on Diodes

- As temperature increases, it adds energy to the diode.

- It reduces the required forward bias voltage for conduction.

- It increases the amount of reverse saturation current.

- Germanium diodes are more sensitive to temperature variations than silicon or gallium arsenide diodes.

### Temperature Dependence of Reverse Saturation Current

Reverse saturation current approximately doubles for every 10°C rise in temperature:

Is ≈ Is0 × 2((T - T_0) / 10)

### Effect on Forward Bias Characteristics

- In the forward bias region, the silicon diode characteristic shifts to the left (lower voltage) at a rate of approximately 2.5 mV per °C increase in temperature.

- Forward bias voltage decreases with increasing temperature.

## 3.9 Breakdown Mechanisms

### 1. Avalanche Breakdown

- Occurs in lightly doped diodes where the depletion region is wide.

- A very large electric field at the junction ruptures covalent bonding, generating a large number of charge carriers — this results in avalanche multiplication.

- Temperature characteristic: Positive — requires more voltage at higher temperature.

### 2. Zener Breakdown

- Occurs in heavily doped diodes where the depletion region is narrow.

- Even a small reverse voltage at the junction ruptures covalent bonding and generates a large number of charge carriers — this is the Zener mechanism.

- Temperature characteristic: Negative — requires less voltage at higher temperature.

| Property | Zener Breakdown | Avalanche Breakdown |
| --- | --- | --- |
| Doping Level | Heavily doped | Lightly doped |
| Depletion Width | Narrow | Wide |
| Voltage Required | Low (< 5 V typically) | High |
| Temperature Coefficient | Negative | Positive |

## 3.10 Diode Capacitance

Depending upon the biasing condition, two types of capacitive effects exist in diodes:

### A. Junction Capacitance (Depletion Capacitance) — Cj

- Most significant capacitance; varies with reverse bias voltage applied.

- Associated with the depletion region of the diode.

- As reverse bias voltage increases, depletion region widens, so junction capacitance decreases.

The junction capacitance is given by:

Cj = (ε × A) / W

| ε | Permittivity of the dielectric |
| --- | --- |
| A | Area of the junction |
| W | Width of the depletion region |

### B. Diffusion Capacitance — Cd

- Appears in a forward-biased PN junction due to storage of charge in the diffusion region.

- As forward bias increases, the depletion region narrows and the concentration of injected charge carriers increases.

- This rate of change of injected charge with applied voltage is defined as diffusion capacitance.

- Increases with forward current; significant only when the diode is forward-biased.

- Typically, less than junction capacitance in most applications.

The diffusion capacitance is given by:

Cd = (τ × I_D) / VT

| τ | Carrier lifetime |
| --- | --- |
| I_D | Diode current |
| V_T | Thermal voltage |

## 3.11 Equivalent Model of Diode

The forward voltage VF applied across the actual diode must overcome two quantities:

- Potential barrier V₀

- Internal voltage drop If × rf

VF = V₀ + If × rf

### Diode Models Summary

| S.No. | Type | Characteristic / Description |
| --- | --- | --- |
| 1 | Approximate Model | Includes ideal diode + barrier potential V₀ + internal dynamic resistance rf. Forward voltage: VF = V₀ + If × rf |
| 2 | Simplified Model | Includes ideal diode + barrier potential V₀ only (rf is neglected). Suitable for most analysis. |
| 3 | Ideal Model | Diode treated as a perfect switch: zero forward voltage drop, infinite reverse resistance. Used for quick estimates. |

# Topic 4: Applications of PN Junction

## 4.1 Applications Overview

| Application | Diodes | Rectifiers | LEDs |
| --- | --- | --- | --- |
| Function | Allow current in one direction only | Convert AC to DC | Emit light when forward biased |
| Application | Signal processing, protection | Power supply circuits | Displays, indicators |

## 4.2 Rectifier Circuits

A rectifier converts an AC signal to a DC signal. A complete power supply system includes:

Transformer: Steps down the high voltage AC from the mains to a lower, usable voltage level (e.g., 230 V AC → 12 V AC).

Rectifier: Converts AC to a unipolar (one-direction) output using diodes.

Filter: Converts the pulsating rectified output to a nearly constant DC using capacitors.

Regulator: Reduces the ripple and stabilizes the DC output voltage.

## 4.3 Half-Wave Rectifier

Uses a single diode. Only the positive half-cycle of AC passes through; the negative half is blocked.

### Key Components and Functions

- Transformer: Steps down AC voltage to a usable level.

- Signal Diode (Rectifier): Acts as a one-way valve. Allows current only when the anode is positive relative to the cathode (Forward Bias). Clips off the negative half of the AC cycle.

- Filter Capacitor: Stores energy during peaks and releases it during troughs, smoothing the pulsating DC into a nearly constant voltage.

- Load: The component that consumes DC power. Output voltage is measured across this load.

### Half-Wave Rectifier — Performance

Ripple Factor (γ): 1.21 (High ripple — poor quality DC)

Maximum Efficiency (η): 40.6%

Output Frequency: Same as input frequency (fin)

PIV Rating: Vm (peak input voltage)

## 4.4 Full-Wave Rectifier

Utilizes both half-cycles of the AC waveform. Two main configurations exist:

### Centre-Tapped Full-Wave Rectifier vs. Bridge Full-Wave Rectifier

| Feature | Centre-Tapped Full-Wave | Bridge Full-Wave |
| --- | --- | --- |
| Number of Diodes | 2 | 4 |
| Transformer | Requires Centre-Tapped (bulkier & expensive) | Standard (smaller & cheaper) |
| Peak Inverse Voltage (PIV) | 2Vm | Vm |
| Transformer Utilization (TUF) | ~0.693 (low) | ~0.812 (higher) |
| Output Frequency | 2 × fin | 2 × fin |
| Max Efficiency | 81.2% | 81.2% |
| Cost | Higher (specialized transformer) | Lower (standard components) |
| Suitability | Low voltage, high current | General-purpose DC power supplies |

## 4.5 Rectifier Performance Metrics

### 1. Ripple Factor (γ)

Measures the amount of AC component (unwanted noise) remaining in the DC output. A lower value is better.

γ = √[(Vrms / Vdc)² - 1]

- Half-Wave: γ = 1.21

- Full-Wave: γ = 0.48

### 2. Rectifier Efficiency (η)

The ratio of DC output power to applied AC input power:

η = (Pdc / Pac) × 100%

- Half-Wave: max 40.6%

- Full-Wave: max 81.2%

### 3. Peak Inverse Voltage (PIV)

The maximum reverse voltage a diode must withstand without breakdown:

- Half-Wave: Vm

- Full-Wave (Centre-Tap): 2Vm

- Full-Wave (Bridge): Vm

## 4.6 Summary: Half-Wave vs. Full-Wave Rectifier

| Parameter | Half-Wave Rectifier | Full-Wave Rectifier (Bridge & Centre-Tap) |
| --- | --- | --- |
| Ripple Factor (γ) | 1.21 (High ripple, poor DC quality) | 0.48 (Low ripple, smoother DC) |
| Max Efficiency (η) | 40.6% | 81.2% |
| Output Frequency | fin (same as input, e.g., 50 Hz) | 2 × fin (double, e.g., 100 Hz) |
| PIV Rating | Vm (peak voltage) | Bridge: Vm ¦ Centre-Tap: 2Vm |
| Number of Diodes | 1 | Bridge: 4 ¦ Centre-Tap: 2 |
| Transformer Type | Standard | Bridge: Standard ¦ Centre-Tap: Special (bulkier) |

# Topic 5: Diode Clippers and Clampers

## 5.1 Diode Clippers (Limiters)

A clipper is a circuit that removes (clips) a portion of the input signal — positive half, negative half, or both — without distorting the remaining part. It is used for protection or waveform shaping.

### Types of Clippers

### 1. Series Clipper

- The diode is placed in series with the load resistor.

- When the diode is OFF: no current flows, output is zero (clipped).

- When the diode is ON: input signal passes through to the output.

### 2. Shunt (Parallel) Clipper

- The diode is placed in parallel with the load.

- When the diode is ON: it short-circuits the signal to ground (clips it).

- When the diode is OFF: signal passes through to the load.

## 5.2 Diode Clampers (DC Restorers)

A clamper is a circuit that shifts the entire waveform up or down by adding a DC component to the signal. Unlike a clipper, it does NOT change the shape of the waveform — it only changes the DC level (baseline).

- Key Component Required: A capacitor is essential for clamper operation.

### Types of Clampers

### 1. Positive Clamper

- Shifts the waveform upward.

- The negative peak of the signal is clamped to zero (or a reference voltage).

- The entire waveform sits above the x-axis.

### 2. Negative Clamper

- Shifts the waveform downward.

- The positive peak is clamped to zero.

- The entire waveform sits below the x-axis.

| Feature | Clipper | Clamper |
| --- | --- | --- |
| Function | Removes part of waveform | Shifts waveform up or down |
| Waveform Shape | Changed (distorted) | Unchanged (shape preserved) |
| Key Component | Diode (+ resistor) | Diode + Capacitor (essential) |
| Output Level | Clipped at threshold | DC shifted |

# Topic 6: Solved Numerical Examples

## 6.1 Thermal Voltage Calculation

Example 1: At a temperature of 27°C, determine the thermal voltage VT.

Given: T = 27°C, k = 1.38 × 10⁻²³ J/K, q = 1.6 × 10⁻¹⁹ C

Solution:

T_K = 273 + 27 = 300 K

VT = (k × TK) / q = (1.38 × 10⁻²³ × 300) / (1.6 × 10⁻¹⁹) ≈ 25.875 mV ≈ 26 mV

## 6.2 Series Diode Circuit — Forward Biased

Example 2: For a series diode configuration with E = 8 V and R = 2.2 kΩ (Silicon diode), determine VD, VR, and ID.

Solution: The applied voltage is sufficient to turn the silicon diode ON.

VD = 0.7 V (barrier potential of silicon)

VR = E − VD = 8 − 0.7 = 7.3 V

ID = VR / R = 7.3 V / 2200 Ω ≈ 3.32 mA

## 6.3 Series Diode Circuit — Insufficient Voltage

Example 3: For a series Silicon diode with E = 0.5 V and R = 1.2 kΩ, determine VD, VR, and ID.

Solution: The applied voltage (0.5 V) is insufficient to turn the silicon diode ON (requires 0.7 V).

ID = 0 A

VR = ID × R = 0 A × 1200 Ω = 0 V

VD = E = 0.5 V (all voltage appears across the diode)

## 6.4 Dual Diode Circuit — Finding Current

Example 4: In a circuit with E₁ = 24 V, E₂ = 4 V, R = 2 kΩ, D1 (forward biased, Si) and D2 (reverse biased), find current I.

Solution: D1 is ON; D2 is OFF (acts as open circuit).

I = (E₁ − E₂ − V0) / R = (24 − 4 − 0.7) / 2000 = 19.3 / 2000 = 9.65 mA

## 6.5 Ideal Diode Circuit

Example 5: A circuit has ideal diodes D1 (reverse biased) and D2 (forward biased), with a 10 V supply, R₁ = 2 Ω, R₂ = 3 Ω, R₃ = 2 Ω. Find current through R₁.

Solution: D1 is reverse biased (open circuit); D2 is forward biased (short circuit for ideal diode).

Reff = R₁ + R₃ = 2 + 2 = 4 Ω

I through R₁ = V / Reff = 10 / 4 = 2.5 A

## 6.6 Shockley Equation — Germanium Diode

Example 6: A germanium diode has a reverse saturation current I₀ = 0.19 μA. Find the current when forward biased with 0.3 V at 27°C. (n = 1 for Ge)

Given: Io = 0.19 μA, VD = 0.3 V, T = 27°C, n = 1

Step 1: Calculate thermal voltage:

VT = TK / 11600 = (273 + 27) / 11600 = 300 / 11600 = 25.862 mV

Step 2: Apply Shockley's equation:

ID = 0.19 × 10⁻⁶ × (e(0.3 / (1 × 25.862×10⁻³)) − 1) ≈ 20.72 mA

## 6.7 Forward Bias Voltage Calculation — Silicon Diode

Example 7: The forward current in a Silicon diode is 15 mA at 27°C. If reverse saturation current is 0.24 nA, find the forward bias voltage. (n = 2 for Si)

Given: ID = 15 mA, Io = 0.24 nA, T = 27°C, n = 2

VT = 300 / 11600 = 25.862 mV

VD = n × VT × ln(ID/Io + 1) = 2 × 25.862 × 10⁻³ × ln(15m/0.24n + 1) ≈ 0.928 V

## 6.8 Static and Dynamic Resistance — Germanium Diode

Example 8: Calculate the static and dynamic resistance of a PN Junction diode when VD = 0.25 V for a Germanium diode. Io = 1 μA, T = 300 K, n = 1.

Given: Io = 1 μA, VD = 0.25 V, T = 300 K, n = 1

VT = 300 / 11600 = 25.862 mV

ID = 1μA × (e(0.25/(1×25.862m)) − 1) = 15.782 mA

(a) Static Resistance:

RD = VD / ID = 0.25 / 15.782m = 15.84 Ω

(b) Dynamic Resistance:

rd = n × VT / ID = 1 × 25.862m / 15.782m = 1.638 Ω

# Topic 7: Quick Reference — Key Formulae and Definitions

## 7.1 Key Formulae Summary

| Quantity | Formula | Notes |
| --- | --- | --- |
| Diode Current (Shockley) | ID = Is(e(VD/nVT) − 1) | n=1 (Ge), n=2 (Si) |
| Thermal Voltage | VT = kTK/q | ≤26 mV at 27°C |
| DC (Static) Resistance | RD = VD / ID | At a specific operating point |
| AC (Dynamic) Resistance | rd = 26mV / ID (for n=1) | Slope of I-V curve |
| Junction Capacitance | Cj = εA / W | Reverse biased |
| Diffusion Capacitance | Cd = τID / VT | Forward biased |
| Ripple Factor | γ = √[(Vrms/Vdc)² − 1] | HW: 1.21, FW: 0.48 |
| Rectifier Efficiency | η = Pdc/Pac × 100% | HW: 40.6%, FW: 81.2% |
| Reverse Current vs. Temperature | Is ≈ Is0 × 2((T−T_0)/10) | Doubles every 10°C |

## 7.2 Key Definitions at a Glance

Semiconductor: A material with electrical conductivity between conductors and insulators; conductivity can be controlled by doping or external fields.

Doping: The deliberate addition of impurities to a pure semiconductor to alter its electrical properties.

Depletion Region: A region at the PN junction devoid of free charge carriers, containing only immobile ions.

Barrier Potential (Vb): The built-in potential that opposes the flow of majority carriers across the junction; 0.7 V for Si, 0.3 V for Ge.

Forward Bias: Positive voltage applied to p-region; reduces barrier potential and allows current flow.

Reverse Bias: Positive voltage applied to n-region; increases barrier potential and blocks current flow.

Knee Voltage: The minimum forward voltage beyond which the forward current rises exponentially.

Rectifier: A circuit that converts AC to DC using diodes.

Clipper: A circuit that removes part of the input waveform without distorting the remainder.

Clamper: A circuit that shifts the DC level of a waveform without changing its shape.

Ripple Factor: A measure of AC content in the DC output of a rectifier; lower is better.

PIV (Peak Inverse Voltage): Maximum reverse voltage a diode must withstand without breaking down.